******************************************************************************

*                                                                            *

* 68060 Upgrade Software Guide                                               *

*                                                                            *

* Copyright Ralph Schmidt                                                    *

*                                                                            *

* Phase5                                                                     *

*                                                                            *

* Rev 2 - Rev 1                                                                      *

*

* Forgot to mention the AFB_68060 Bit

*

* Rev 1

*                                                                            *

*                                                                            *

* EMail: laire@uni-paderborn.de                                              *

*                                                                            *

******************************************************************************

0) Phase5 will sell the first 060 accelerator,

namely the Cyberstorm 060, rather soon and wants to achieve a smooth

integration into the Amiga.

Therefore we hope that software companys and pd programmers

will adopt their software to deal with the 68060.

1) 68060 Flag in ExecBase->AttnFlags

;exec/execbase.i

BITDEF	AF,FPU40,6	; Set if 68040/68060 FPU

BITDEF	AF,68060,7	; Set if 68060

;exec/execbase.h

#define	AFF_68060	(1L<<7)

2) Inststructions supported by the 68040 but not by the 68060 directly that

can hurt performance:

o divx.l (64bit divide)

o mulx.l (64bit multiply)

o fdbcc

o fscc

o movep

o There are other instructions that are not supported or have to

be emulated but these aren't relevant for user applications.

3) Instructions supported by the 68060 in hardware that are emulated on

the 68040:

o fint

o fintrz

4) Emulation limitations

The emulation can't reliably emulate unsupported instructions

that access the (ssp) in a way that it contradicts the basic

definition of a stack. (ssp)=sp in supervisor mode.

I don't think anybody is using such constructs in his software

but I want to mention it to stop anybody from using these:

div{u,s}.l		-(ssp),dr:dq

mul{u,s}.l		-(ssp),dr:dq

f<op>.p			-(ssp),fpn

f<op>.p			fpn,(ssp)+

f<op>.{b,w,l,s,d,x}	-(ssp),fpn

fs<cc>.b		-(ssp)

fmovem.x		-(ssp),dn

fmovem.x		dn,(ssp)+

f<op>.x			fpn,(ssp)+	;Underflow,SNAN exception

f<op>.{b,w,l}		fpn,(ssp)+	;Enabled OPERR

5) Other software problems.

o Don't use Aztec C.

It is not compatible with the 68060.

Popular applications with this problem: CED 2.02,CED 3.5,Mand2000D

*I fixed this by patching Supervisor()*,

so don't worry about your programs but

don't use it in future software if possible.

Here's the bad Code:

btst.b	#4,$129(a6)		;check for 68881 flag in AttnFlags

beq	1$			;skip if not

lea	2$,a5

jsr	-30(a6)			;do it in supervisor mode

bra	1$

2$

clr.l	-(sp)

frestore (sp)+			;reset the ffp stuff

;The 68060 FRestore stackframe is 12 bytes long but it only created

;a NULL frame with the size of 4 Bytes so the Stack is wrong afterwards.

;RTE->jump into unknown regions...crash

rte				;and return

6) New Stackframe format

These informations are only interesting for Debugger programmers

and kernel hackers.

o 3-Byte in the 1st Longword = 0 then NULL Stackframe.

xxxx00xx	;1st Longword of the FPU stackframe

xxxxxxxx	;2nd Longword of the FPU stackframe

xxxxxxxx	;3rd Longword of the FPU stackframe

xxxxxxxx	;PC

xxxx	;SR

d0-a6	;Registers

o 3-Byte in the 1st Longword !=0 then BUSY Stackframe.

ffffffff	;Busy Fake Longword

xxxxxxxx	;FPCR

xxxxxxxx	;FPSR

xxxxxxxx	;FPIAR

fp0-fp7	;FPU Registers

xxxxxxxx	;1st Longword of the FPU stackframe

xxxxxxxx	;2nd Longword of the FPU stackframe

xxxxxxxx	;3rd Longword of the FPU stackframe

xxxxxxxx	;PC

xxxx	;SR

d0-a6	;Registers

7) MMU Table differences

The 68040 and 68060 mmu tables are almost the same but there is

an important difference. For the 68060 the MMU-Tables have to

be placed into non-cached ram!!!!!!!

So if you manipulate the mmu-table you have to mark the pages

with your added tables as non-cacheable.

Another difference is that the Cyberstorm software uses the

area $ffff8000-$ffffffff for itself.

You are NOT allowed to change the mmu-table that this area

is touched in any way. This can lead to serious crashes.

8) Barfly 1.20 is the first Assembler/Debugger that supports the

68060. It can be found on aminet:dev/asm/barfly1_20.lha